NEVADA BUREAU OF MINES AND GEOLOGY (NBMG)				2004

NBMG Open-File Report 04-1 - dBase III+ file version

Nevada Oil and Gas Well Database (NVOILWEL)

Compiled by Ronald H. Hess

Assisted by Shane P. Fitch and Sean N. Warren

1 JUNE 2004

This information should be considered preliminary.    It has not been edited or checked for completeness or accuracy.

NVOILWEL was originally produced in 1986 by Ronald H. Hess.

If you have questions about this data set please contact Ron Hess at (775) 784-6691 ext. 121 (email rhess@unr.edu) or David Davis at (775) 784-6691 ext. 133 (email ddavis@unr.edu).

This is the dBase III+ digital file that contains the data used to develop NBMG open-file report 04-1. It contains the data found in the Nevada Bureau of Mines and Geology Oil and Gas Well Database (NVOILWEL). Nevada oil and gas well records, logs, and other information located at the Nevada Bureau of Mines and Geology Information office.

UTM location information plotted from approximate township and range locations at 1:100,000 scale. UTM coordinates are in zone 11, NAD 27 projection.

Nevada Oil and Gas Well Database field descriptions

API:		American Petroleum Institute well identification number.

COUNTY:	County that well is located in.

PERMIT:	State of Nevada permit number. Only issued for wells drilled since 1954.

OPERATOR:	Name of operator that drilled the well or current operator of a producing or injection well.

WELL:		Well name.

SECTION:	Section number.

TOWNSHIP:	Township number with letter designating either north(N) or south(S) township.

RANGE:	Range number with letter designating east(E) range.

PARTSECT:	Quarter section location information.

DISTFROMLI:Distance in feet from section line.

PERMIT_ISS:Date that State drilling permit was issued.

COMPL_DATE:Date well was completed.

STATUS:	Current well status.

TD:		Total depth of well in feet.

ELEVATION: Elevation of well in feet.

TD_NUMERIC:Total depth of well, in feet, in a numeric field.

TOPS:		Depth, in feet, to geologic tops, significant geologic units, markers, or lithology.

AL_BOT:	Depth to bottom of alluvium, 99999 = no data.

LOGS:		List of lithology and electric logs, with interval logged in feet, housed at the Nevada Bureau of Mines and Geology Information office.

SAMPLES:	List of cuttings and core intervals housed at the Nevada Bureau of Mines and Geology Information office.

SHOW:		Indication of oil, gas, or excessive water shows.

OIL_FIELD: Field name for production or injection well in established field.

REMARKS:	General information about a well, drilling problems, missing records, initial production figures, questions or clarifications about data contained in the above fields, etc.

APROX_UTMN:Approximate UTM northing location, zone 11, NAD 27, meters

APROX_UTME:Approximate UTM easting location, zone 11, NAD 27, meters

Data sources include the public oil and gas records depository at Nevada Bureau of Mines and Geology, consisting of drilling permits, drilling reports, completion reports, lithologic and drillers logs, electric logs, professional papers, other open-file reports, and various company transmittals. Data are far from complete for many well records.

Nevada Oil and Gas Well Database field Abbreviations

STATUS						TESTS

A - Abandoned					DST - Drill Stem Test

P & A - Plugged and Abandoned		PT - Production Test

Conf. - Confidential

D & A - Drilled and Abandoned		LOGS

J & A - Junked and Abandoned

NYD - Not Yet Drilled			BHCS - Bore Hole Comp. Sonic

Cal - Caliper

CNL - Compensated Neutron Log

AGES						CN - Compensated Neutron

DI - Dual Induction Log

Prec. - Precambrian			DIL - Dual Induction Laterolog

Pal. - Paleozoic			DLL - Dual Laterolog

Mes. - Mesozoic				DM - Dipmeter

Cen. - Cenozoic				ES - Electric Log

Camb. - Cambrian			IES - Induction Electric Log

Ord. - Ordovician			FDL - Formation Density Log

Sil. - Silurian				FDC - Comp. Formation Density

Dev. - Devonian				FIL - Fracture Indication Log

Miss. - Mississippian			FIL - Fracture Ident. Log

Penn. - Pennsylvanian			GR - Gamma Ray

Perm. - Permian				LL - Laterolog

Tri. - Triassic				ML - Microlog/Minilog

Jur. - Jurassic				MLL - Microlaterolog

Cret. - Cretaceous			N - Neutron

Tert. - Tertiary			PL - Proximity

Quat. - Quaternary			PML - Proximity Microlog

S - Sonic/Acoustilog

FORMATIONS					LSS - Long Spaced Sonic

SNP - Sidewall Porosity Log

SP - Spontaneous Potential

TS - Temperature Survey

CBL - Cement Bond Log

Lithology		Formal	Informal

Group			Gp.		gp.

Formation		Fm.		fm.

Member		Mbr.		mbr.			REMARKS

Sandstone		Ss.		ss.			ft - feet

Siltstone		Sts.		sts.			BPM - barrels per minute

Shale			Sh.		sh.			DF - derrick floor

Limestone		Ls.		ls.			KB - kelly brushing

Dolomite		Dol.		dol.			TD - total depth

Conglomerate	Cgl.		cgl.			GPM - gallons per minute

Quartzite		Qtz.		qtz.			BWPD - barrels water per day

Volcanics		Volc.		volc.			MCF - thousand cubic feet

Fanglomerate	Fgl.		fgl.			bbls. - barrels

NBMG - Nevada Bureau of Mines and Geology